TEM Protocol for Cells Attached to a Substrate

- Pour off medium and replace with phosphate buffer containing 2% paraformaldehyde + 2.5% glutaraldehyde.

- Incubate for 1h at room temperature (RT).

- After 1h fixation, wash the fixed cells twice with phosphate buffer. Degenerating and necrotic cells will generally spontaneously lift off the substrate and can be discarded if so desired.

- Incubate with phosphate buffer containing 1% osmium tetroxide, 1.5% potassium ferricyanide, and 0.2% tannic acid for 1h at RT in the dark.

- Dehydrate in an ascending series of alcohols: 25% ethanol (1×, 10 min), 50% ethanol containing 1% uranyl acetate (for en bloc staining; 1×, 60 min), 75% ethanol (1×, 10 min), 95% ethanol (3×, 10 min), 100% ethanol (3×, 10 min).

- Remove the final 100% ethanol from the wells using a glass pipette, replace it with propylene oxide, and leave it for 30s.

- Draw the propylene oxide up and down in the pipette and transfer the cells to a microfuge tube. The cells will come off in sheets of transparent film. Centrifuge at 9,500g at room temperature for 5 min.

- Subject pellets to two more changes of propylene oxide (10 min each), centrifuging each time.

- Incubate cell pellets with a 1:1 mixture of propylene oxide: epoxy resin on a rotary mixer for 6h at room temperature.

- Incubate cell pellets with a 1:2 mixture of propylene oxide: epoxy resin on a rotary mixer for 6h at room temperature.

- Incubate cell pellets with 100% epoxy resin on a rotary mixer for 6h at room temperature.

- Incubate cell pellets with 100% epoxy resin on a rotary mixer for 2h at room temperature.

- Polymerize in an oven at 70°C for at least 12h.

- Remove the sample from the centrifuge tube, cut off the pellet, and place it in the tip of a BEEM capsule.

- Fill the BEEM capsule with 100% epoxy resin and polymerize in an oven at 70°C for at least 12h.

| Epoxy Resin |  |
| --- | --- |
| Embed 812 | 10.0 g |
| DDSA | 4.5 g |
| NMA | 6.0 g |
| DMP-30 | 0.35 ml |

Modified from Graham, L., & Orenstein, J.M. (2007). Processing tissue and cells for transmission electron microscopy in diagnostic pathology and research. Nature Protocols, 2(10), 2439-2450.